Please Note:

This template is intended for educational purposes only, without any express or implied warranty of any kind, including warranties of accuracy, completeness, or fitness for any particular purpose.  You agree that your use of the template is without any recourse whatsoever to PG&E, SCE, SDG&E, or their affiliates. The template is a draft, and anyone using this document should seek the advice of an attorney to develop appropriate ordinance language to meet its jurisdiction’s specific needs, as state and local laws may differ.

This document is the product of a collaborative effort between Peninsula Clean Energy, Silicon Valley Clean Energy, and the Local Energy Codes team.

Please contact the Codes and Standards Local Energy Codes Team at info@LocalEnergyCodes.com for additional information.

This program is funded by California utility customers and administered by Pacific Gas and Electric Company, San Diego Gas & Electric Company (SDG&E®), and Southern California Edison Company under the auspices of the California Public Utilities Commission and in support of the California Energy Commission.

# 2025 Existing Single Family

# Flexible Path Compliance (FlexPath)

# Model Reach Code

Version 2.1

October 2025

## Context

This document is intended to be used in conjunction with companion materials which are available at LocalEnergyCodes.com and BayAreaReachCodes.com. These materials provide important information that is necessary to develop, adopt and implement the reach code.

## Scope

- All single family buildings that undergo certain improvements must include a set of energy efficiency, renewable energy and/or electrification measures that meet a specified energy-savings target.

- The energy savings target is expressed as a score and individual measures are assigned points weighted by site-energy savings.  The target and points are specific to each climate zone and building vintage.

- To include the electric readiness policy, use the Flex Path, Readiness and Cool Roof  model code at LocalEnergyCodes.com.

## Instructions

Jurisdictions may wish to modify elements of the ordinance, such as those marked in blue text and those marked with comments. When modifying the language, ensure all references are maintained and that the ordinance still meets the CEC requirements. Also, edit the Purpose section to reflect the changes.

Amendments to the State Code appear in strikeouts (deletions) and underlines (additions). Text in blue is for staff reference and should be removed from the final ordinance.

The draft ordinance text is provided as an example only and intended as a starting point for customization for creating an ordinance that aligns with state and local requirements.  Ensure all ordinance materials are reviewed and verified by relevant jurisdiction staff and the city/county attorney.

## Changes from Prior Versions

Please check LocalEnergyCodes.com to obtain the latest version of this document and supporting materials.  This document will be updated frequently to comport with new features in the Cost Effectiveness Explorer.

| Since Version | Change |
| --- | --- |
| 2.0 | Corrected reference to heat pump space conditioner specifications in Table 150.0-K. Isolated FlexPath from readiness and cool roof model code language. Reformatted. Revised definition and deleted exception to accommodate changes to vintages. Deleted exception for amended definition of New Construction. Updated specifications for high efficiency split systems. Clarified electric readiness requirements for FlexPath PV credit versus mandated upon electrical work. Consolidated exceptions. Added references to requirements in Section 150.2. |
| 1.10 and prior | Added comment to limit application to older vintages if needed as a result of the repeal of the investment tax credit for solar PV. Corrected redline in exception to 150.0(w) and Attachment 1. Clarified duct insulation options for different climate zones. Corrected heat pump clothes dryer specification in Table 150.0-K. Removed “All Electric Home” from measures. Corrected references to 2022 Energy Code. Modified electric readiness for PV credit under FlexPath. Added outdoor appliances to electric readiness. Added links to new resources. Updated measure specifications for water heating and cool roof packages. Updated specifications for insulation measures. Updated links to state agency and program resources. |

ORDINANCE NO. _________

AN ORDINANCE OF THE [GOVERNING BODY]

OF THE [CITY/TOWN/COUNTY] OF [JURISDICTION]  AMENDING [IDENTIFY SUBPARTS, INCLUDING CHAPTERS, DIVISIONS, ETC.] OF THE [JURISDICTION] CODE TO REQUIRE HIGHER ENERGY PERFORMANCE FOR CERTAIN SINGLE FAMILY PROJECTS AND ADOPTING REQUIRED FINDINGS

Recitals and Findings

- See companion materials for guidance.

Section 100.0 is modified to add a new section (i) as follows:

(i) Single Family Building Remodel Energy Reach Code - Purpose and Intent.

In addition to all requirements of the California Energy Code applicable to Single Family building additions and alterations, the energy efficiency and renewable energy measures specified in Section 150.0(w) shall be required for certain single family additions and alterations.

Section 100.1(b) is modified by adding the following definition:

COVERED SINGLE FAMILY PROJECT shall mean any project in a Single Family residential building originally permitted for construction before 2011…

[Option 1] that meets any of the following criteria:

1.	Any change to an existing building that increases conditioned floor area by [XX] or more square feet in a one-year period.

2.	Any project that includes an addition and alteration whose altered components cover [ZZ] square feet or greater in a one-year period.

3.	Installation or replacement of an air conditioner.

[Option 2] that requires an electrical permit, a mechanical permit and a plumbing permit, with the exception of projects with a total valuation of less than [Valuation trigger], or includes installation or replacement of an air conditioner.

[Option 3] that includes an addition, of any size or value, or alteration of such a structure with a building permit valuation of [Valuation trigger] or higher or includes installation or replacement of an air conditioner.

[Option 4] with a building permit valuation of [Valuation trigger] or higher and that includes any of the following:

1.	Any additions, or any change, rearrangement or addition, other than a repair, of the structural elements of an existing building including foundations, footing, sub-floors, lintels, beams, columns, girders, slabs, roof trusses, staircases, load bearing walls, door frames, window frames, or any other part of the building that resists force or moment.

2.	Change or rearrangement of the plan configuration of walls and full-height partitions of an existing building.

3.	Modification of the electrical system, heating or cooling equipment or gas plumbing.

[Option 5] includes an addition, alteration, or remodel or the alteration to such a structure that affects a floor area which exceeds [square footage trigger] of the existing floor area of the structure or has a combined valuation of [valuation trigger] or more or includes installation or replacement of an air conditioner. When any changes are made in the building, such as walls, columns, beams or girders, floor or ceiling joists and coverings (subfloor and drywall), roof rafters, roof diaphragms, foundations, piles or retaining walls or similar components, the floor area of all rooms affected by such changes shall be included in computing floor areas for purposes of applying this definition. This definition does not apply to project scopes that are solely limited to any of the following: the replacement and upgrading of residential roof coverings, exterior wall finishes and/or floor finishes; alterations that are limited to providing access for persons with disabilities; and additions of [square footage minimum] square feet or less.

The first two paragraphs of Section 150.0 SINGLE-FAMILY RESIDENTIAL BUILDINGS – MANDATORY FEATURES AND DEVICES are modified to read as follows:

Single-family residential buildings shall comply with the applicable requirements of Sections 150.0(a) through 150.0(v).

NOTE: The requirements of Sections 150.0(a) through 150.0(v) apply to newly constructed buildings. Sections 150.2(a) and 150.2(b) specify which requirements of Sections 150.0(a) through 150.0(v) also apply to additions or alterations. In addition, Covered Single Family Projects shall also be required to comply with Section 150.0(w).

A new Section, (w), is added to Section 150.0 as follows:

(w)	A Covered Single Family Project shall install a set of measures based on the building vintage from the Measure Menu Table, Table 150.0-J, [add reference to table(s) for additional climate zones, if applicable] to achieve a total Measure Point Score that is equal to or greater than the Target Score in Table 150.0-I [add reference to table(s) for additional climate zones, if applicable].  In addition, all mandatory measures listed in Table 150.0-J shall be installed.  Measure verification shall be explicitly included as an addendum to the Certificate of Compliance to be filed pursuant to 2025 Title 24, Part 6, Section 10-103.

Installed measures shall meet the specifications in Table 150.0-K. Building vintage is the year in which the original construction permit for the building was submitted, as documented by building department records, or the permit issue date of an addition or alteration that satisfied the Performance Standards (California Energy Code, Title 24, Part 6, Section 150.1(b)) that were in effect at that time. Unless otherwise specified, the requirements shall apply to the entire dwelling unit, not just the additional or altered portion. Measures from the Measure Menu table that are to be installed to satisfy requirements under the California Energy Code, Title 24, Part 6, may not count towards compliance with these requirements.  Where these requirements conflict with other California Energy Code requirements, the stricter requirements shall prevail.

Exception 1 to 150.0(w):  Creation of a new accessory dwelling unit or junior accessory dwelling unit that is within the existing space of a single family dwelling or accessory structure and includes an expansion of not more than 150 square feet beyond the same physical dimensions as the existing accessory structure. An expansion beyond the physical dimensions of the existing accessory structure shall be limited to accommodating ingress and egress. Or, if the project would not otherwise be a Covered Single Family Project were it not for the inclusion of an accessory dwelling unit or junior accessory dwelling unit that meets the criteria above.

Exception 2 to 150.0(w): Mobile Homes, Manufactured Housing, or Factory-built Housing as defined in Division 13 of the California Health and Safety 12 Code (commencing with Section 17000 of the Health and Safety Code).

Exception 3 to Section 150.0(w): The project is solely related to any of the following: a repair as defined by Title 24 Part 2 Section 202; medically necessary improvements; seismic safety improvements; safety improvement to remove a known hazard; roofing projects; fenestration projects; or Emergency Housing pursuant to Appendix P of the California Building Code.

Exception 4 to 150.0(w): If the applicant demonstrates, using Commission-certified compliance software as specified by Section 10-109(c) and Section 10-116, that the Energy Budget of the Proposed Building Design would be less than or equal to the Energy Budget of the building under the project if it included any set of measures that would achieve compliance under this Section 150.0(w).

[Optional Exceptions related to “Covered Projects”]

Exception 5 to 150.0(w):  A Covered Single Family Project, other than an addition, that would not otherwise be subject to this section 150.0(w) but for installation of solar PV, solar water heating, EV charging, electrical upgrades for solar PV or EV charging, or energy storage.

[Optional Exceptions Related to Measures and Compliance]

Exception 6 to 150.0(w): If the dwelling unit has previously installed measures from the Measure Menu, Table 150.0-I, and compliance can be demonstrated to the [appropriate title] or designee, then these measures shall not be required to be newly installed, and appropriate credit shall be included in the applicable compliance calculations.

Exception 7 to 150.0(w): A measure that is necessary for compliance is prohibited because of a covenant or other deed restriction on the property, such as a homeowners association covenant.

Exception 8 to 150.0(w): The applicant may request an exemption to any requirements of this chapter which would impair the historic integrity of any building listed on a local, state, or federal register of historic structures, as determined by the [appropriate title] or designee and as regulated by the California Historic Building Code (Title 24, Part 8). In making a determination of exemption, the Planning Director or designee may require the submittal of an evaluation by an architectural historian or similar expert.

Exception 9 to 150.0(w): Due to conditions specific to the project, it is technically or economically infeasible to achieve compliance, the [appropriate title] or designee may reduce the Target Score and/or waive some or all of the mandatory requirements.

[Exception Related to hardship or infeasibility]

Exception 10 to 150.0(w):

[Option 1]

If the project includes circumstances which constitute hardship or infeasibility, the applicant may request an exemption. In applying for an exemption, the burden is on the applicant to show hardship or infeasibility. Circumstances that constitute hardship or infeasibility shall include one or more of the following:

a.	That the cost of achieving compliance exceeds [valuation limit] of the valuation of cost of the project;

b.	That it is technically infeasible to achieve compliance through all packages due to conditions specific to the project;

c.	That strict compliance with these standards would create or maintain a hazardous condition(s) and present a life safety risk to the occupants.

1.	Application. Based on the following, the applicant shall identify in writing the specific requirements of the standards for compliance that the project is unable to achieve and the circumstances that make it a hardship or infeasible for the project to comply with this chapter. The applicant may not petition for relief from any requirement of the 2025 California Energy Code (Title 24, Part 6) and referenced standards, or the 2025 California Green Building Standards (Title 24, Part 11) of the California Building Standards Code. Granting of exemption. If the [appropriate title] or designee determines that it is a hardship or infeasible for the applicant to fully meet the requirements of this chapter and that granting the requested exemption will not cause the building to fail to comply with the 2025 California Energy Code (Title 24, Part 6) and referenced standards, or the 2025 California Green Building Standards (Title 24, Part 11) of the California Building Standards Code, the authority having jurisdiction shall determine the minimum feasible threshold of compliance reasonably achievable for the project. If an exemption is granted, the applicant shall be required to comply with this chapter in all other respects and shall be required to achieve the threshold of compliance determined to be achievable by the [appropriate title] or designee.

2.	Denial of exemption. If the chief building official determines that it is reasonably possible for the applicant to fully meet the requirements of this chapter, the request shall be denied, and the applicant shall be notified of the decision in writing. The project and compliance documentation shall be modified to comply with the standards for compliance.

3.	Appeal. Any aggrieved applicant or person may appeal the determination of the [appropriate title] or designee regarding the granting or denial of an exemption or compliance with any other provision of this chapter. An appeal of a determination of the [appropriate title] or designee shall be filed in writing with the [appropriate body].

[Option 2]

Expenditures of more than [specify percentage] of the project valuation for a resident owner(s) or owner(s) of a residence occupied by a dependent that can demonstrate that they qualify as a low-income utility customer by being eligible for the California Alternative Rates for Energy (CARE) [or other criterion]. If the least-cost set of measures that would be required for compliance exceeds [specify percentage] of the total project valuation, the Target Score may be reduced by subtracting the points associated with the lowest cost measures first, until the cost of the remaining measures does not exceed 10% [or other amount] of the project valuation.  The project valuation shall exclude any measures that are required under this Section but shall include all measures that are otherwise required under the State Energy Code, Title 24, Part 6. [This exception is recommended if the definition of Covered Single Family Project does not include a valuation.]

[Option3]

An applicant who resides in the dwelling unit and qualifies as a low-income utility customer, or is the owner of the dwelling unit which is occupied by a dependent who qualifies as a low-income utility customer, may comply by either a) installing the duct sealing measure, the lighting measure and water heating package, or b) installing at least 1 kW of solar PV that meets the requirements of 2025 Title 24 Reference Appendix JA11. A low-income utility customer is anyone who is eligible for the California Alternative Rates for Energy (CARE) or Family Electric Rate Assistance Program (FERA) program [or other criterion].

Table 150.0-I: TARGET SCORES

| Building Vintage | Pre-1978 | 1978-1991 | 1992-2010 |
| --- | --- | --- | --- |
| Climate Zone [A] |  |  |  |
| Climate Zone [B] |  |  |  |

Table 150.0-J: MEASURE MENU [, Climate Zone A]

| ID | Measures | Building Vintage |  |  |
| --- | --- | --- | --- | --- |
|  |  | Pre-1978 | 1978-1991 | 1992-2010 |
| E1 | Lighting Measures | Mandatory |  |  |
| E2 | Water Heating Package |  |  |  |
| E3 | Air Sealing |  |  |  |
| E4.A | R-38 Attic Insulation |  |  |  |
| E4.B | R-49 Attic Insulation |  |  |  |
| E5 | Duct Sealing |  |  |  |
| E6.A | New Ducts, R-6 Insulation + Duct Sealing [CZ 3, 5, 6, 7 only] |  |  |  |
| E6.B | New Ducts, R-8 Insulation + Duct Sealing |  |  |  |
| E7 | Windows |  |  |  |
| E8 | Wall Insulation |  |  |  |
| E10.A | R-19 Raised floor insulation |  |  |  |
| E10.B | R-30 Raised floor insulation |  |  |  |
| E11 | Cool Roof [CZ 6-15 only] |  |  |  |
| E12 | Radiant Barrier Under Roof |  |  |  |
| FS1 | Heat Pump Water Heater Replacing Gas |  |  |  |
| FS2 | High Eff. Heat Pump Water Heater Replacing Gas |  |  |  |
| FS3 | Heat Pump Water Heater Replacing Electric |  |  |  |
| FS4 | High Eff. Heat Pump Water Heater Replacing Electric |  |  |  |
| FS5 | Heat Pump Space Conditioning System |  |  |  |
| FS6 | High Eff. Heat Pump Space Conditioning System |  |  |  |
| FS7 | Dual Fuel Heat Pump Space Conditioning System |  |  |  |
| FS8 | Heat Pump Clothes Dryer |  |  |  |
| FS9 | Induction Cooktop |  |  |  |
| PV | Solar PV [and Electric Readiness] |  |  |  |

Table 150.0-K: MEASURE SPECIFICATIONS

| ID | Measure Specification |
| --- | --- |
|  | Energy Efficiency Measures |
| E1 | Lighting Measures – Install lighting with an efficiency of 45 lumens per watt or greater in all interior and exterior screw-in fixtures. Install photocell, occupancy sensor or energy management system controls that meet the requirements of 150.0(k)3 in all outdoor lighting permanently mounted to a residential building or to other buildings on the same lot. |
| E2 | Water Heating Package: Insulate all accessible hot water pipes with pipe insulation a minimum of ¾ inch thick. This includes insulating the supply pipe leaving the water heater, piping to faucets underneath sinks, and accessible pipes in attic spaces or crawlspaces. Upgrade fittings in sinks and showers to meet current California Green Building Standards Code (Title 24, Part 11) Section 4.303 water efficiency requirements. Exception: Upgraded fixtures are not required if existing fixtures have rated or measured flow rates of no more than ten percent greater than 2025 California Green Building Standards Code (Title 24, Part 11) Section 4.303 water efficiency requirements. |
| E3 | Air Sealing: Seal all accessible cracks, holes, and gaps in the building envelope at walls, floors, and ceilings. Pay special attention to penetrations including plumbing, electrical, and mechanical vents, recessed can light luminaires, and windows. Weather-strip doors if not already present. Verification shall be conducted following a prescriptive checklist that outlines which building aspects need to be addressed by the permit applicant and verified by an inspector. Compliance can also be demonstrated with blower door testing conducted by a certified ECC Rater no more than three years prior to the permit application date that either: a) shows at least a 30 percent reduction from pre-retrofit conditions; or b) shows that the number of air changes per hour at 50 Pascals pressure difference (ACH50) does not exceed ten for Pre-1978 vintage buildings, seven for 1978 to 1991 vintage buildings and five for 1992-2010 vintage buildings. If combustion appliances are located within the pressure boundary of the building, conduct a combustion safety test by a certified ECC Rater or a professional certified by the Building Performance Institute, in accordance with the BPI Technical Standards for the Building Analyst Professional. |
| E4.A | R-38 Attic Insulation: Attic insulation shall be installed to achieve a weighted assembly U-factor of 0.025 or insulation installed at the ceiling level shall have a thermal resistance of R-38 or greater for the insulation alone. Recessed downlight luminaires in the ceiling shall be covered with insulation to the same depth as the rest of the ceiling. Luminaires not rated for insulation contact must be replaced or fitted with a fire-proof cover that allows for insulation to be installed directly over the cover. Exception: In buildings where existing R-30 is present and existing recessed downlight luminaires are not rated for insulation contact, insulation is not required to be installed over the luminaires. |
| E4.B | R-49 Attic Insulation: Attic insulation shall be installed to achieve a weighted assembly U-factor of 0.020 or insulation installed at the ceiling level shall have a thermal resistance of R-49 or greater for the insulation alone. Recessed downlight luminaires in the ceiling shall be covered with insulation to the same depth as the rest of the ceiling. Luminaires not rated for insulation contact must be replaced or fitted with a fire-proof cover that allows for insulation to be installed directly over the cover. Exception: In buildings where existing R-30 is present and existing recessed downlight luminaires are not rated for insulation contact, insulation is not required to be installed over the luminaires. |
| E5 | Duct Sealing: Air seal all space conditioning ductwork to meet the requirements of the 2025 Title 24, Part 6, Section 150.2(b)1E. The duct system must be tested by a ECC Rater no more than three years prior to the Covered Single Family Project permit application date to verify the duct sealing and confirm that the requirements have been met. This measure may not be combined with the New Ducts and Duct Sealing measure in this Table. Exception: Buildings without ductwork or where the ducts are in conditioned space. |
| E6A | [Climate Zones 3, 5, 6, 7 only (R-8 required prescriptively elsewhere)] New Ducts, R-6 insulation + Duct Sealing: Replace existing space conditioning ductwork with new R-6 ducts that meet the requirements of 2025 Title 24, Part 6, Section 150.0(m)11. This measure may not be combined with the Duct Sealing measure in this Table. To qualify, a preexisting measure must have been installed no more than three years before the Covered Single Family Project permit application date. |
| E6B Alternate | New Ducts, R-8 insulation + Duct Sealing: Replace existing space conditioning ductwork with new R-8 ducts that meet the requirements of 2025 Title 24 Section 150.0(m)11. This measure may not be combined with the Duct Sealing measure in this Table. To qualify, a preexisting measure must have been installed no more than three years before the Covered Single Family Project permit application date. |
| E7 | Windows: Replace at least 50% of existing windows with high performance windows with an area-weighted average U-factor no greater than 0.27. |
| E8 | R-15 Wall Insulation: Install wall insulation in all exterior walls to achieve a weighted U-factor of 0.095 or install wall insulation in all exterior wall cavities that shall result in an installed thermal resistance of R-15 or greater for the insulation alone. |
| E9 | Reserved for future use |
| E10.A | R-19 Floor Insulation: Raised-floors shall be insulated such that the floor assembly has an assembly U-factor equal to or less than U-0.037 or shall be insulated between wood framing with insulation having an R-value equal to or greater than R-19. |
| E10.B Alternate | R-30 Floor Insulation: Raised-floors shall be insulated such that the floor assembly has an assembly U-factor equal to or less than U-0.028 or shall be insulated between wood framing with insulation having an R-value equal to or greater than R-30. |
| E11 | [Climate Zones 6-15 only. This item can be required as a mandatory measure for a reroofing project. See Decision Guide for details] Cool Roof : Install a cool roof on at least 50% of the roof area. For steep-sloped roofs (ratio of rise to run greater than 2:12) install a roofing product rated by the Cool Roof Rating Council to have an aged solar reflectance equal to or greater than 0.25 and a thermal emittance equal to or greater than 0.75. |
| E12 | Radiant Barrier: A radiant barrier that meets the requirements of Section 150.1(c)2 shall be installed under at least 50% of the roof surface. |
|  | Fuel Substitution and Solar PV Measures |
| FS1 | Heat Pump Water Heater (HPWH) Replacing Gas: Replace existing natural gas water heater with a heat pump water heater that meets the requirements of Sections 110.3 and 150.2(b)1.H.iii.b. |
| FS2 | High Efficiency Heat Pump Water Heater (HPWH) Replacing Gas: Replace existing natural gas water heater with heat pump water heater with a Northwest Energy Efficiency Alliance (NEEA) Tier 3 or higher rating that also meets the requirements of Sections 110.3 and 150.2(b)1.H.iii.c. |
| FS3 | Heat Pump Water Heater (HPWH) Replacing Electric: Replace existing electric resistance water heater with a heat pump water heater that meets the requirements of Sections 110.3 and 150.2(b)1.H.iii.b. |
| FS4 | High Efficiency Heat Pump Water Heater (HPWH) Replacing Electric: Replace existing electric resistance water heater with heat pump water heater with a Northwest Energy Efficiency Alliance (NEEA) Tier 3 or higher rating that also meets the requirements of Sections 110.3, and 150.2(b)1.H.iii.c. |
| FS5 | Heat Pump Space Conditioning System: Replace all existing gas and electric resistance primary space heating systems with a heat pump system that meets the requirements of Sections 110.2, 150.2(b)1.C, 150.2(b)1.E, 150.2(b)1.F, and 150.2(b)1.G. |
| FS6 | High Efficiency Heat Pump Space Conditioning System: Replace all existing gas and electric resistance primary space heating systems with an electric-only heat pump system that meets the requirements of Sections 110.2 and 150.2(b)1.C, 150.2(b)1.E, 150.2(b)1.F, and 150.2(b)1.G and one of the following: - A ducted heat pump system with a SEER2 rating of 16.5 or greater, an EER2 rating of 12.48 or greater and an HSPF2 rating of 9.5 or greater; or - A ductless mini-split heat pump system with a SEER2 rating of 16.0 or greater, an EER2 rating of 12.0 or greater and an HSPF2 rating of 9.0 or greater |
| FS7 | Dual Fuel Heat Pump Space Conditioning System: Install a heat pump space conditioning system that meets the requirements of Sections 110.2 and 150.2(b)1.C, 150.2(b)1.E, 150.2(b)1.F, and 150.2(b)1.G and either: - Replaces all existing gas and electric resistance primary heating systems with a hybrid gas and electric heat pump system, or - Is an electric-heat pump system in tandem with a gas furnace and controls to use the gas furnace for backup heat only. |
| FS8 | Heat Pump Clothes Dryer: Replace existing electric resistance or gas clothes dryer with heat pump dryer with no resistance element and cap gas line. |
| FS9 | Induction Cooktop: Replace all existing gas and electric resistance stove tops with induction stove top and cap the gas line. |
| PV.A | Solar PV: Install a solar PV system that meets the requirements of Section 150.1(c)14. |
| PV.B Alternate | Solar PV + Electric Readiness: Install a solar PV system that meets the requirements of Section 150.1(c)14. In addition, for existing PV systems that had been installed prior to the application date of the current project, meet at least two [or more] of the electric readiness requirements in Section 150.0(x). |

A new Section, (x), is added to Section 150.0 as follows:

(x) Electric Readiness for Credit for Solar PV Measure in Table 150.0-K

1.	Electric range. The electrical components shall include either of the following:

A.	A 125 volt, 20 amp electrical receptacle that is connected to the electric panel with a 120/240 volt 3 conductor branch circuit rated at 50 amps minimum, within 3 feet from the appliance and accessible to the appliance with no obstructions. Both ends of the unused conductor shall be labeled with the word “spare” and be electrically isolated. Space shall be reserved for a single pole circuit breaker in the electrical panel adjacent to the circuit breaker for the branch circuit and labeled with the words “Future Use”.

B.	A pathway for a future 240 volt 50 amp minimum branch circuit that shall consist of either conductors or raceway from the main electrical service panel. The main electric panel shall have space reserved to allow for the installation of a double pole circuit breaker for a future electric range installation. The reserved space shall be permanently marked as “For Future 240V use”. The raceway or conductors shall terminate at a junction box within 3 feet of the appliance. The blank cover shall be identified as “240V ready”.

2.	Electric dryer. The electrical components shall include either of the following:

A.	A dedicated 125 volt, 20 amp electrical receptacle that is connected to the electric panel with a 120/240 volt 3 conductor branch circuit rated at 30 amps minimum, within 3 feet from the appliance and accessible to the appliance with no obstructions. Both ends of the unused conductor shall be labeled with the word “spare” and be electrically isolated. Space shall be reserved for a single pole circuit breaker in the electrical panel adjacent to the circuit breaker for the branch circuit and labeled with the words “Future Use”; or,

B.	A pathway for a future 240 volt 30 amp minimum branch circuit that shall consist of either conductors or raceway from the main electrical service panel. The main electric panel shall have space reserved to allow for the installation of a double pole circuit breaker for a future heat pump dryer installation. The reserved space shall be permanently marked as “For Future 240V use”. The raceway or conductors shall terminate at a junction box within 3 feet of the appliance. The blank cover shall be identified as “240V ready”.

3.	Heat pump water heater. The electrical components shall include either of the following:

A.	A dedicated 125 volt, 20 amp electrical receptacle that is connected to the electric panel with a 120/240 volt 3 conductor, 10 AWG copper branch circuit rated at 30 amps minimum, within 3 feet from the water heater and accessible to the water heater with no obstructions. Both ends of the unused conductor shall be labeled with the word “spare” and be electrically isolated. Space shall be reserved for a single pole circuit breaker space in the electrical panel adjacent to the circuit breaker for the branch circuit and labeled with the words “Future 240V Use”; or

B.	A pathway for a future 240 volt 30 amp minimum branch circuit that shall consist of either conductors or raceway from the main electrical service panel. The main electric panel shall have space reserved to allow for the installation of a double pole circuit breaker for a future HPWH installation. The reserved space shall be permanently marked as “For Future 240V use”. The pathway shall terminate at a junction box within 3 feet of the appliance. The blank cover shall be identified as “240V ready”.

4.	Outdoor gas appliances. Install infrastructure and reserve physical space to accommodate future installation of an electric equivalent of that system that serves the same function, as certified by a registered design professional or licensed electrical contractor.

A.	Install conduit designed to serve a future electric appliance(s) with the same function, including the appropriate voltage, phase, minimum amperage, and an electrical receptacle or junction box within five feet of the appliance that is accessible with no obstructions, in accordance with manufacturer requirements and the California Electrical Code. In lieu of or in addition to conduit, electrically isolated branch circuit wiring may be installed; and

B.	Label both ends of the unused conduit or conductors “For Future Electrical Appliance”; and

C.	Reserve circuit breakers in the electrical panel(s) for each branch circuit, appropriately labeled; and

D.	Designate physical space for future electric appliances, including equipment footprint, on the construction drawings. The footprint necessary for future electric appliances may overlap with the location of currently designed combustion equipment.

Exception to Section 150.0(w)4: Generator systems used for emergency power generation.

- 5.	Battery Storage. Meet the requirements A through D below.

- A.	At least one of the following shall be provided:

- i.	ESS ready interconnection equipment with a minimum backed up capacity of 60 amps and a minimum of four ESS supplied branch circuits, or

- ii.	A dedicated raceway from the main service to a panelboard (subpanel) that supplies the branch circuits in 150.0(s)2. All branch circuits are permitted to be supplied by the main service panel prior to the installation of an ESS. The trade size of the raceway shall be not less than one inch. The panelboard that supplies the branch circuits (subpanel) must be labeled “Subpanel shall include all backed-up load circuits.”

- B.	A minimum of four branch circuits shall be identified and have their source of supply collocated at a single panelboard suitable to be supplied by the ESS. At least one circuit shall supply the refrigerator, one lighting circuit shall be located near the primary egress, and at least one circuit shall supply a sleeping room receptacle outlet.

- C.	The main panelboard shall have a minimum busbar rating of 225 amps or sufficient capacity a battery storage system and circuits in paragraphs A and B above.

- D.	Sufficient space shall be reserved to allow future installation of a system isolation equipment/transfer switch within 3 feet of the main panelboard. Raceways shall be installed between the panelboard and the system isolation equipment/transfer switch location to allow the connection of backup power source.

6.	All-Electric Plan.  Provide a plan, including single line diagrams and calculations for electrification of the equipment below. Calculations shall be in accordance with California Electrical Code Article 220.83 or 220.87..

- A.	All appliances with gas stub outs including space conditioning, water heating, range, and clothes dryer;

- B.	Low Power Level 2 EV charger; and

- C.	Battery storage system rated at 240 volts, 60 amp minimum.

The first paragraph of Section 150.2(a) is modified to read as follows:

(a) Additions. Additions to existing single-family residential buildings shall meet the requirements of Sections 110.0 through 110.9, Sections 150.0(a) through (n), (p), (q), (w), and either Section 150.2(a)1 or 2.

The first paragraph of Section 150.2(b) is modified to read as follows:

(b) Alterations. Alterations to existing single-family residential buildings or alterations in conjunction with a change in building occupancy to a single-family residential occupancy shall meet the requirements of Section 150.0(w) and either Item 1 or 2 below.

OTHER ORDINANCE SECTIONS

Severability. If any word, phrase, sentence part, section, subsection or other portion of this amendment or any application thereof to any person or circumstance is declared void, unconstitutional, or invalid for any reason, then such word, phrase, sentence, part, section, subsection, or other portion, or the prescribed application thereof, shall be severable, and the remaining provisions of this amendment, and all applications thereof, not having been declared void, unconstitutional or invalid, shall remain in full force and effect.

CEQA. This ordinance is categorically exempt from CEQA because it is an action taken by a regulatory agency for the purpose of protecting the environment (CEQA Guidelines Section 15308). In addition, this ordinance is exempt from CEQA under the general rule, 15061(b)(3), on the grounds that these standards are more stringent than the State energy standards, there are no reasonably foreseeable adverse impacts, and there is no possibility that the activity in question may have a significant effect on the environment. The following findings are made in support of these determinations:

1.	The purpose of the implementation of a Reach Code is to reduce the amount of greenhouse gas emissions in the [Jurisdiction] that are produced from buildings.

2.	The Reach Code approval process requires that the [Jurisdiction] determines that the local standards will require buildings to be designed to consume less energy than current statewide requirements. Furthermore, the California Energy Commission approval process requires that the [Jurisdiction] make the findings as part of its approval process. Therefore, the Reach Code standards can only go into effect if they protect the environment by making buildings more efficient.

Violations. Violation of the requirements of this Chapter shall be considered an infraction of the [jurisdiction Municipal/County Code], punishable by all the sanctions prescribed in [cite local reference to infractions].

Effective Date. This ordinance shall become effective and enforceable as of [Date], upon approval of the California Energy Commission or upon the date the California Building Standards Commission (CBSC) accepts the ordinance for filing, whichever is later.